Iron Sight

Museum Shooting Gallery — Technical Handover

Raspberry Pi Zero W2 + Pi Camera NoIR + IR Bar

# System Overview

This is a plug-and-play interactive shooting gallery for museum installation. A physical rifle prop aims at a projected game. An IR sensor bar provides two infrared reference points. A Pi Camera NoIR mounted on the rifle detects those IR dots and translates their position into crosshair coordinates. The game runs in a browser and receives coordinates over a local WiFi WebSocket.

| IR Sensor Bar | Pi Camera NoIR | Browser Game |
| --- | --- | --- |
| USB-powered. Placed at screen edge. Emits two IR dots. | Mounted on rifle barrel. Detects IR dots via OpenCV. | shooting-gallery.html open in Chrome. Receives X,Y via WebSocket. |

Data flow: IR bar → Camera → Pi Zero W2 (OpenCV + WebSocket server) → WiFi → Browser game

# Hardware

| Main computer | Raspberry Pi Zero W2 |
| --- | --- |
| Camera | Pi Camera v2 NoIR (IMX219, no IR filter) |
| IR source | USB Wii sensor bar (2 IR LEDs) |
| Connection | WiFi (same LAN as game browser) |
| Power | 5V micro USB — min 2.5A recommended |
| Camera cable | 22-pin mini CSI ribbon (Zero W2 specific — NOT standard 15-pin) |

ℹ  The Pi Zero W2 uses a smaller 22-pin mini CSI connector. Do NOT use the standard 15-pin ribbon cable from Pi 3/4.

# SD Card & OS Setup

The SD card already has a working Raspberry Pi OS Lite 64-bit image with all dependencies installed. If you need to reflash:

## Flash a fresh SD card

- Download Raspberry Pi Imager from raspberrypi.com/software

- Select OS: Raspberry Pi OS Lite (64-bit)

- Click the gear icon before writing to set: hostname, username/password, WiFi SSID+password, enable SSH

- Recommended hostname: irrifle  (makes URL predictable: irrifle.local)

- Recommended user: admin

## If the existing SD card is used

Just plug it in. WiFi credentials and all software are already installed. The hostname is currently:

fraspberry

ℹ  To change the hostname to something more descriptive: sudo hostnamectl set-hostname irrifle

# GPIO Wiring

The 25 MHz clock signal on GPIO4 is required for the IR camera module. It is enabled automatically at boot via /boot/firmware/config.txt — no manual steps needed.

## Camera connection

Connect the Pi Camera v2 NoIR ribbon cable to the CSI port on the Pi Zero W2. Lift the plastic clip, insert the ribbon with contacts facing away from the board, press clip down.

## IR sensor bar

Power the USB IR sensor bar from any USB port or USB power bank. Point it at the projection screen edge. No data connection needed — it just emits infrared light.

# Software — What Is Installed

| python3-opencv | OpenCV 4.10 — finds IR dots in camera frames |
| --- | --- |
| python3-numpy | Required by OpenCV |
| websockets | Python WebSocket server (pip, user install) |
| libcamera-apps | rpicam-vid — streams camera frames |
| avahi-daemon | mDNS — enables hostname.local addressing |
| i2c-tools | Diagnostic tools (not used in production) |

## config.txt entries

These lines are already in /boot/firmware/config.txt:

dtparam=audio=off

dtoverlay=gpclk,pin=4,freq=25000000

ℹ  The gpclk overlay outputs 25 MHz on GPIO4 at boot. This is required for the IR camera module to respond on I2C. Do not remove these lines.

# ir_detect.py — The Main Script

This script does everything: reads the camera, finds IR dots, calculates aim position, and broadcasts over WebSocket. It lives at /home/admin/ir_detect.py.

## Full script

import cv2

import numpy as np

import subprocess

import asyncio

import websockets

import json

import threading

latest_pos = {'x': 0.5, 'y': 0.5, 'shoot': False}

clients = set()

async def handler(websocket):

global clients

clients.add(websocket)

print(f'Client connected: {websocket.remote_address}')

try:

async for message in websocket:

pass

except:

pass

finally:

clients.discard(websocket)

print('Client disconnected')

async def broadcast():

global clients

while True:

if clients:

msg = json.dumps(latest_pos)

dead = set()

for ws in clients.copy():

try:

await ws.send(msg)

except:

dead.add(ws)

clients -= dead

await asyncio.sleep(0.033)  # ~30fps

async def main():

async with websockets.serve(handler, '0.0.0.0', 8765):

print('WebSocket server running on port 8765')

await broadcast()

def camera_loop():

global latest_pos

cmd = [

'rpicam-vid', '--width', '640', '--height', '480',

'--framerate', '30', '--codec', 'mjpeg',

'--output', '-', '--timeout', '0', '--nopreview'

]

process = subprocess.Popen(cmd, stdout=subprocess.PIPE,

stderr=subprocess.DEVNULL)

buf = b''

while True:

chunk = process.stdout.read(4096)

if not chunk:

break

buf += chunk

start = buf.find(b'\xff\xd8')

end = buf.find(b'\xff\xd9')

if start != -1 and end != -1 and end > start:

jpg = buf[start:end+2]

buf = buf[end+2:]

frame = cv2.imdecode(np.frombuffer(jpg, np.uint8),

cv2.IMREAD_COLOR)

if frame is None:

continue

gray = cv2.cvtColor(frame, cv2.COLOR_BGR2GRAY)

_, thresh = cv2.threshold(gray, 200, 255,

cv2.THRESH_BINARY)

contours, _ = cv2.findContours(thresh,

cv2.RETR_EXTERNAL, cv2.CHAIN_APPROX_SIMPLE)

dots = []

for c in contours:

area = cv2.contourArea(c)

if area > 3:

M = cv2.moments(c)

if M['m00'] > 0:

cx = int(M['m10'] / M['m00'])

cy = int(M['m01'] / M['m00'])

dots.append((area, cx, cy))

dots.sort(reverse=True)

dots = dots[:2]

if len(dots) == 2:

_, x1, y1 = dots[0]

_, x2, y2 = dots[1]

if x1 > x2:

x1, y1, x2, y2 = x2, y2, x1, y1

nx = ((x1 + x2) // 2) / 640

ny = ((y1 + y2) // 2) / 480

latest_pos['x'] = round(nx, 3)

latest_pos['y'] = round(ny, 3)

print(f'aim: ({nx:.3f}, {ny:.3f})')

print('Starting IR tracker + WebSocket server...')

cam_thread = threading.Thread(target=camera_loop, daemon=True)

cam_thread.start()

asyncio.run(main())

# Running the System

## Start the tracker manually

ssh admin@irrifle.local

python3 /home/admin/ir_detect.py

You should see:

Starting IR tracker + WebSocket server...

WebSocket server running on port 8765

aim: (0.312, 0.445)   # once camera sees IR bar

## Make it start automatically on boot

To run ir_detect.py automatically when the Pi powers on:

sudo nano /etc/systemd/system/irsensor.service

Paste this content:

[Unit]

Description=IR Sensor WebSocket Server

After=network.target

[Service]

ExecStart=/usr/bin/python3 /home/admin/ir_detect.py

WorkingDirectory=/home/admin

Restart=always

User=admin

[Install]

WantedBy=multi-user.target

Then enable it:

sudo systemctl daemon-reload

sudo systemctl enable irsensor

sudo systemctl start irsensor

ℹ  After this, the IR tracker starts automatically every time the Pi powers on. No SSH needed for day-to-day operation.

# The Game — shooting-gallery.html

The game is a single self-contained HTML file. Open it in Chrome on any machine on the same WiFi network as the Pi. The crosshair follows the IR bar automatically.

## WebSocket connection in the HTML

Near the bottom of the script section, this block connects to the Pi:

const ws = new WebSocket('ws://irrifle.local:8765');

ws.onopen = () => console.log('IR tracker connected');

ws.onmessage = e => {

const d = JSON.parse(e.data);

mx = d.x * window.innerWidth;

my = d.y * window.innerHeight;

ch.style.left = mx + 'px';

ch.style.top  = my + 'px';

if (d.shoot) shoot(mx, my);

};

ws.onerror = () => console.warn('IR tracker not connected - using mouse');

ws.onclose = () => console.warn('IR tracker disconnected');

ℹ  If the Pi hostname changes, update the ws:// line. Using hostname.local avoids needing to know the IP address.

## Game controls

| Mouse | Aims crosshair (fallback when IR not connected) |
| --- | --- |
| Click | Shoot |
| Spacebar | Shoot at current crosshair position |
| d.shoot (WebSocket) | Fire signal from Pi (for physical trigger — not yet wired) |

## Game config — top of script

These values can be tweaked to adjust difficulty:

const CFG = {

duration: 30,       // round length in seconds

minOnScreen: 1,     // minimum simultaneous targets

maxOnScreen: 2,     // maximum simultaneous targets

minSize: 70,        // smallest target diameter (px)

maxSize: 150,       // largest target diameter (px)

minLife: 3500,      // target stays on screen min (ms)

maxLife: 6000,      // target stays on screen max (ms)

spawnTick: 1800,    // check for new target every N ms

hitsToDestroy: 3,   // hits needed to destroy a target

};

## Background image (projection mapping)

The background is currently solid black. To swap in a projected image or scene:

In the <style> section, find this block and uncomment the background line:

body {

background: #000;

/* ── BACKGROUND IMAGE ────────────────────

background: url('your-image.jpg') center/cover no-repeat;

─────────────────────────────────────── */

}

Replace your-image.jpg with the path to your scene image.

# Troubleshooting

## Crosshair not moving

- Check ir_detect.py is running on the Pi

- Check browser console (F12) for WebSocket errors

- Confirm Pi and game browser are on the same WiFi network

- Try: ssh admin@irrifle.local — if this fails, the Pi is not reachable

## ir_detect.py shows 'dots found: 0' or '1'

- Move camera further from the IR bar — dots merge when too close

- Dim or block ambient IR sources (direct sunlight, some LED lights)

- Check the IR bar is powered (USB) and facing the camera

- Try lowering the threshold in the script from 200 to 180

## Camera not found

- Check ribbon cable is fully seated in the CSI connector

- Check config.txt has: dtoverlay=gpclk,pin=4,freq=25000000

- Run: rpicam-still -o test.jpg — should capture without errors

## WebSocket connects but aim is wildly off

The X/Y coordinates are normalised 0.0–1.0 from the camera frame. If the camera is not level or is at an angle to the IR bar, the mapping will be skewed. Fix by:

- Mounting camera parallel to the screen plane

- Ensuring IR bar dots are both clearly visible in the camera frame

- (Future) Implementing a 4-corner homography calibration

## Pi not reachable at hostname.local

# On Mac/Linux, check mDNS:

ping irrifle.local

# If not found, find IP via router admin page

# Then SSH directly by IP:

ssh admin@192.168.x.x

# Once in, check avahi:

sudo systemctl status avahi-daemon

sudo systemctl enable avahi-daemon

sudo systemctl start avahi-daemon

# Next Steps / Known Limitations

## Physical trigger

The d.shoot field in the WebSocket message is always false — no physical trigger is wired yet. Options:

- Wire a momentary switch between a Pi GPIO pin and GND

- Detect press in ir_detect.py and set latest_pos['shoot'] = True for one frame

- Alternative: spacebar on keyboard as temporary stand-in

## Calibration

Currently, aim position is the midpoint between the two IR dots, normalised to frame dimensions. A proper 4-corner calibration (aim at each screen corner, record dot positions, compute homography) would significantly improve accuracy for non-centered setups.

## Recoil

A solenoid linear actuator is available for recoil simulation. Wire to Pi GPIO, trigger on shoot event.

## Performance on Zero W2

The Pi Zero W2 is slower than the Pi 3. If frame processing is sluggish, reduce resolution in ir_detect.py:

# Change these lines in camera_loop():

'--width', '320', '--height', '240',   # was 640x480

# And update normalisation:

nx = ((x1 + x2) // 2) / 320   # was 640

ny = ((y1 + y2) // 2) / 240   # was 480

# Quick Reference

| SSH into Pi | ssh admin@irrifle.local |
| --- | --- |
| Start tracker | python3 /home/admin/ir_detect.py |
| Check service | sudo systemctl status irsensor |
| View logs | journalctl -u irsensor -f |
| Restart service | sudo systemctl restart irsensor |
| Test camera | rpicam-still -o test.jpg |
| Check I2C | sudo i2cdetect -y 1 |
| Pi IP address | hostname -I |
| WebSocket port | 8765 |
| Game file | shooting-gallery.html (open in Chrome) |

Iron Sight — Museum Installation — Technical Handover